# Basic Blinky (Arduino / PlatformIO)

// Feather M4 Express Blinky

const int LED_PIN = 13;

void setup() {

pinMode(LED_PIN, OUTPUT);

}

void loop() {

digitalWrite(LED_PIN, HIGH); // LED ON

delay(500);

digitalWrite(LED_PIN, LOW);  // LED OFF

delay(500);

}

What it does

Turns the LED on for 500 ms

Turns it off for 500 ms

Repeats forever

# Slightly Better Version (Non-Blocking Timer)

This version avoids delay() and is more embedded-style correct.

const int LED_PIN = 13;

unsigned long previousMillis = 0;

const long interval = 500;

bool ledState = LOW;

void setup() {

pinMode(LED_PIN, OUTPUT);

}

void loop() {

unsigned long currentMillis = millis();

if (currentMillis - previousMillis >= interval) {

previousMillis = currentMillis;

ledState = !ledState;

digitalWrite(LED_PIN, ledState);

}

}

Advantages:

Allows other code to run concurrently

Useful for multi-tasking microcontroller projects

# CircuitPython Blinky (Optional)

Since the Feather M4 Express ships with CircuitPython, you can also do:

import board

import digitalio

import time

led = digitalio.DigitalInOut(board.D13)

led.direction = digitalio.Direction.OUTPUT

while True:

led.value = True

time.sleep(0.5)

led.value = False

time.sleep(0.5)

# Quick Setup Checklist

- Install Arduino IDE

- Add Adafruit SAMD boards

- Select Tools → Board → Adafruit Feather M4 Express

- Upload the sketch